# Packet Tracer - Access Control List Demonstration

# Objectives

Part 1: Verify Local Connectivity and Test Access Control List

Part 2: Remove Access Control List and Repeat Test

# Background

In this activity, you will observe how an access control list (ACL) can be used to prevent a ping from reaching hosts on remote networks. After removing the ACL from the configuration, the pings will be successful.

# Instructions

## Verify Local Connectivity and Test Access Control List

### Ping devices on the local network to verify connectivity.

From the command prompt of PC1, ping PC2.

From the command prompt of PC1, ping PC3.

#### Question:

Why were the pings successful?

Type your answers here.

### Ping devices on remote networks to test ACL functionality.

From the command prompt of PC1, ping PC4.

From the command prompt of PC1, ping the DNS Server.

#### Question:

Why did the pings fail? (Hint: Use simulation mode or view the router configurations to investigate.)

Type your answers here.

## Remove ACL and Repeat Test

### Use show commands to investigate the ACL configuration.

Use the show run and show access-lists commands to view the currently configured ACLs. To quickly view the current ACLs, use show access-lists. Enter the show access-lists command, followed by a space and a question mark (?) to view the available options:

R1# show access-lists ?

<1-199>  ACL number

WORD     ACL name

<cr>

If you know the ACL number or name, you can filter the show output further. However, R1 only has one ACL; therefore, the show access-lists command will suffice.

R1# show access-lists

Standard IP access list 11

10 deny 192.168.10.0 0.0.0.255

20 permit any

The first line of the ACL prevents any packets originating in the 192.168.10.0/24 network, which includes Internet Control Message Protocol (ICMP) echoes (ping requests). The second line of the ACL allows all other ip traffic from any source to transverse the router.

For an ACL to impact router operation, it must be applied to an interface in a specific direction. In this scenario, the ACL is used to filter traffic exiting an interface. Therefore, all traffic leaving the specified interface of R1 will be inspected against ACL 11.

- Although you can view IP information with the show ip interface command, it may be more efficient in some situations to simply use the show run command.

#### Question:

- Using one or both of these commands, to which interface and direction is the ACL applied?

Type your answers here.

### Remove access list 11 from the configuration.

You can remove ACLs from the configuration by issuing the no access list [number of the ACL] command. The no access-list command deletes all ACLs configured on the router. The no access-list [number of the ACL] command removes only a specific ACL.

Under the Serial0/0/0 interface, remove access-list 11, previously applied to the interface as an outgoing filter:

R1(config)# int se0/0/0

R1(config-if)#no ip access-group 11 out

In global configuration mode, remove the ACL by entering the following command:

R1(config)# no access-list 11

Verify that PC1 can now ping the DNS Server and PC4.

- Lab Answers

- Part 1

- Step 1

  - Screenshot showing ping from PC1 to PC2

  - Screenshot showing ping from PC1 to PC3

- Step 1ab Question:

- The pings were successful because there is no policy set to filter out ICMP message between the two local networks.

- Step 2

  - Screenshot showing ping from PC1 to PC4

  - Screenshot showing ping from PC1 to DNS Server

- Step 2 ab Question:

- The pings failed because it looks like there is configuration on R1. The configuration is an ACL to deny pings from exiting interface s0/0/0.

- Part 2

- Step 1

  - Screenshot showing show-acl output on R1

- Question b: the interface and direction the ACL is applied is Serial 0/0/0, outgoing traffic.

- Step 2

  - Screenshot showing access list removal on R1

  - Screenshot showing ping from PC1 to PC4 now after access list removal

End of document